| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 1 of 7 |
| --- | --- | --- |

Duties of the Watch Officer / Engineer at Sea and in Port

Duties of the Watch Officer at Sea

1. General Responsibilities

- The officer on watch whilst in charge of the bridge is the Captain’s representative and as such is in effective control of the vessel and all persons on board. This must be his overriding thought in the discharge of his duties that shall have his sole attention. He must exercise the utmost vigilance and ensure the other members of the watch are equally attentive to their duties.

- The officer on watch is responsible to the Captain for the safe navigation of the vessel and for the observance of the practices of good seamanship. He shall handle the vessel at all times in strict accordance with these applicable rules of the road.

- The officer on watch shall be thoroughly familiar and comply in all respects with conditions. He shall promptly consult the Captain if he is doubtful of the interpretation of any of the instructions, or if he has any doubts regarding the manner in which he is required to carry out his duties.

- Relieving the watch

It is the responsibility of the officer relieving the watch to ensure that he has become acquainted with all the information on the steaming situation to understand it clearly and become oriented before relieving the officer on watch. He shall therefore, report early enough to ensure the watch is relieved on time. He shall not take over the watch until his eyes have become fully accustomed to the prevailing conditions of light.

- Turning - over the watch

3.1. The officer on watch shall not turn-over the watch to the relieving officer until assured that he is physically capable and fully understands the conditions as set out by the Captain. If he is not assured, he must call the Captain and report the circumstances to the Captain.

3.2. The watch is not to be relieved during a maneuver.

3.3. Before leaving the bridge, the relieved watch officer shall make the required entries in the deck log book and initial each for the period of his watch.

3.4. After turning-over the watch at 20:00 hours, 24:00 hours, and 04:00 hours, the relieved watch officer and lookout are to make a tour of inspection of the public rooms, laundries, galleys, provision stores ,drying rooms , accommodation, steering gear room  and weather permitting, bosun’s store.  Watchkeepers carrying out rounds at sea are to keep in regular contact with the bridge. Rounds are to be recorded in the log book.

- Heavy Weather

The officer on watch shall be alert to changes in weather or sea state that may create a hazard to personnel, the vessel, or equipment. In approaching heavy weather, he shall have the vessel inspected in good time to maki certain that all gear and stores are secured and those weather doors and ports are tightly closed. He shall alert the Captain to the changing conditions, and take timely action to discontinue work in areas that could become hazardous with a change in weather or sea state conditions.

- When the Captain is to Be Called

The officer on watch shall notify the Captain immediately whenever he is in doubt about a navigational safety matter at any time the vessel appears to be standing into danger, or in any other situation as set forth in the standing orders.

- Notifications to the engine room

6.1. The officer on watch shall notify the engineer on watch as follows:

- At least one hour before the end of sea passage;

- When proceeding at full sea speed and it becomes apparent that changes in speed may be required due to weather or sea conditions including approaching low visibility, heavy traffic, or in an emergency situation;

- Any change in ambient temperature that would affect the operation of the deck machinery;

- Unusual concentrations of ice, seaweed, other marine life or shallow water that might affect intakes to the engine room; and

- When appropriate to check that the bridge and engine room clocks are synchronized.

| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 2 of 7 |
| --- | --- | --- |

- Danger Bearings

7.1. When it is practical to do so, danger bearings shall be used to determine the safe maneuvering limits of the vessel’s track.

7.2. Points of land to be used for radar ‘danger bearings’ shall be carefully selected to ensure positive identification, clearly defined points such as beacons, isolated small islands etc., are preferred.

7.3. Two fixed objects onshore often form natural ‘leading lines’ or ‘ranges’. The use of such leading lines is useful in making ‘good’ the intended track. Buoys or other floating objects shall not be used unless no other choice is useful in making ‘good’ the intended track. Buoys or other floating objects shall not be used unless no other choice is available.

- Manual / Automatic Steering

8.1. The steering mode shall be changed-over to manual steering in the following situations:

- Whenever the Captain’s instructions specify manual steering;

- At all planned navigation alterations of course;

- For all traffic avoidance maneuvers;

- Whenever the dept. of water is not sufficient for the automatic steering; and

- For any emergency situation.

8.2. Except in a sudden emergency situation, manual steering shall not be done by the officer on watch at any time.

- All deck officers shall be thoroughly familiar with the operation of the steering systems on the vessel and in particular with the methods of changing-over the steering mode.

- All changing-over of the steering mode must be performed by the officer on watch. The unlicensed personnel employed as helmsman must never be allowed to change-over, or interfere with, the steering mode controls.

- Whenever the vessel is being steered by an automatic steering system the officer(s) on watch shall always be aware of the possibility of sudden failure of the system and allow for this accordingly. The ‘off course alarm’ unit must always be activated in order to give warning of any failure of the automatic steering system.

- Anchor Watch

Whenever a vessel is at anchor, the Bridge shall be manned by an OOW. Any other bridge manning arrangement at anchor shall be subject to a documented risk assessment and establishment of additional controls to mitigate the risk. Master shall, at his discretion, increase the number of personnel on watch in areas/locations where there is a risk of unauthorized persons gaining access to the vessel e.g. Piracy or Stowaways or on account of bad weather and/or strong tides. Whenever the vessel is working cargo at anchor, appropriate arrangements shall be made by the Master to ensure that a safe anchor watch is being maintained.

A continuous navigational watch shall be maintained at anchorage. While at anchor, the Duty Officer shall:

- Check at frequent intervals that the anchor is holding, by checking the positions and entering time and bearing in Log Book. Proper use of radar and GPS guard rings/alarms shall be made. Bearing of buoys shall be avoided if other means are available to ascertain the vessel’s position. Ship’s position circle, showing maximum reach of the vessel as she swings around at change of tide and exact position of the anchor shall be charted on the largest scale chart available or on a scale diagram.

- If there is doubt that the vessel in not maintaining position, the OOW shall:

- Inform Master, inform Engine Room

- Inform relevant coastal/port authorities.

- Call anchor stations.

- Use engines prudently to reduce the hazard due to the vessel dragging anchor.

- In congested anchorage, keep adequate number of fenders ready and rigged.

- Ensure that safe watch keeping is maintained and safety rounds conducted throughout the night.

- Ensure that the state of readiness of the main engine is maintained to allow for manoeuvring at short notice. Any maintenance work to be carried out on the M/E must be in agreement with the office.

- Ensure that ship/shore communication system is operational and manned. A continuous listening watch shall be maintained on VHF, on the appropriate working channel for the area and on channel 16.

- Ensure that the vessel exhibits the appropriate lights/shapes and that sound signals are given when the sight is reduced.

- Keep course recorder operating at all times while the vessel is anchored.

| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 3 of 7 |
| --- | --- | --- |

- Observe weather, tidal and sea conditions and record same in the Deck Logbook. Closely monitor any changes in weather conditions. Take early and effective action in case of deteriorating weather, deteriorating visibility or when a dangerous situation starts to develop. Notify Master about the deteriorating of weather or the development of a dangerous situation. While awaiting arrival of the Master on the bridge, the officer in charge of the navigational watch shall take all necessary measures required immediately to avert an incident. (Master may evaluate the situation and consider paying out more cable to ease the strain and main engines may also be used to ease the strain on the cable. Putting to sea in the advent of adverse and severe weather instead of remaining at anchor is usually a safer option.)

- In the event that anchor vessel anchored in the vicinity is observed to be dragging anchor or another vessel navigating in the vicinity is coming close to own vessel. OOW shall while awaiting arrival of the Master on the bridge, take all necessary measures required immediately to avert an incident, including:

- Inform Master, Engine Room/Duty Engineer and port authorities;

- Notify the dragging vessel or vessel navigating in the vicinity, by any means of communication, regarding the impending danger;

- Have the engines ready;

- Call anchor stations,

- Keep adequate number of fenders ready and rigged on the side vessel is expected to come closer;

- Make use of helm/engines to cant vessel away;

- Arrange to heave up the anchor and clear the area, if danger appears to be increasing.

- Plot ships in the immediate vicinity, on the navigational chart.

- Consider factors affecting a vessel when at anchor; including yawing and snatching.

- Take the necessary measures if an emergency has or might occur.

- Call the Master if orders are received or if changes in weather, position or other circumstances so require.

Visual Inspection of the Anchors and Anchoring Equipment shall be conducted at every opportunity by the Anchor Team. Such inspection shall include visual check of the anchors, chain cable, joining shackle, swivel, windlass, brake lining, gypsy, and chain cable as far as practicable. Any failures or pre-cursory signs off failure shall be reported promptly to the Master. In case of brake lining wastage of more than 25% from original, the brake lining should be replaced.OOW must complete checklist YS-531-03-08 “Anchoring and Anchor Watch Checklist” prior taking over watch at anchorage.

Duties of the Watch Officer in Port

- General Responsibilities

1.1 On any ship safely moored or safely at anchor under normal circumstances in port, the master shall arrange for an appropriate and effective watch to be maintained for the purpose of safety.

1.2 Arrangements for keeping a deck watch when the ship is in port shall at all times be adequate to:

(1) Ensure the safety of life, of the ship, the port and the environment, and the safe operation of all machinery related to cargo operation;

(2) Observe international, national and local rules; and

(3) Maintain order and the normal routine of the ship.

1.3 The master shall decide the composition and duration of the deck watch depending on the conditions of mooring, type of the ship and character of duties.

1.4 A qualified officer shall be in charge of the deck watch.

1.5 The necessary equipment shall be so arranged as to provide for efficient watchkeeping.

1.6 The chief engineer, in consultation with the master, shall ensure that engineering watchkeeping arrangements are adequate to maintain a safe engineering watch while in port. When deciding the composition of the engineering watch, which may include appropriate engine-room ratings, the following points are among those to be taken into account:

(1) There shall always be an officer in charge of the engineering watch;

(2) Officers, while in charge of an engineering watch, shall not be assigned or undertake any task or duty which would interfere with their supervisory duty in respect of the ship's machinery system.

| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 4 of 7 |
| --- | --- | --- |

2.  Relieving the watch

2.1 Officers in charge of the deck or engineering watch shall not hand over the watch to their relieving officer if they have any reason to believe that the latter is obviously not capable of carrying out watchkeeping duties effectively, in which case the master or chief engineer shall be notified accordingly. Relieving officers of the deck or engineering watch shall ensure that all members of their watch are apparently fully capable of performing their duties effectively.

2.2 If, at the moment of handing over the deck or engineering watch, an important operation is being performed it shall be concluded by the officer being relieved, except when ordered otherwise by the master or chief engineer.

3.  Taking over the deck watch

3.1 Prior to taking over the deck watch, the relieving officer shall be informed of the following by the officer in charge of the deck watch as to:

(1) The depth of the water at the berth, the ship's draught, the level and time of high and low waters; the securing of the moorings, the arrangement of anchors and the scope of the anchor chain, and other mooring features important to the safety of the ship; the state of main engines and their availability for emergency use;

(2) All work to be performed on board the ship; the nature, amount and disposition of cargo loaded or remaining, and any residue on board after unloading the ship;

(3) The level of water in bilges and ballast tanks;

(4) The signals or lights being exhibited or sounded;

(5) The number of crew members required to be on board and the presence of any other persons on board;

(6) The state of fire-fighting appliances;

(7) Any special port regulations;

(8) The Master's standing and special orders;

(9) The lines of communication available between the ship and shore personnel, including port authorities, in the event of an emergency arising or assistance being required;

(10) Any other circumstances of importance to the safety of the ship, its crew, cargo or protection of the environment from pollution; and

(11) The procedures for notifying the appropriate authority of any environmental pollution resulting from ship activities.

3.2 Relieving officers, before assuming charge of the deck watch, shall verify that:

(1) The securing of moorings and anchor chain is adequate;

(2) The appropriate signals or lights are properly exhibited or sounded;

(3) Safety measures and fire protection regulations are being maintained;

(4) They are aware of the nature of any hazardous or dangerous cargo being loaded or discharged and the appropriate action to be taken in the event of any spillage or fire;

(5) No external conditions or circumstances imperil the ship and that it does not imperil others.

- Performing the deck watch

4.1 The officer in charge of the deck watch shall:

(1) Make rounds to inspect the ship at appropriate intervals;

(2) Pay particular attention to:

- the condition and securing of the gangway, anchor chain and moorings, especially at the turn of the tide and in berths with a large rise and fall, if necessary, taking measures to ensure that they are in normal working condition,

- the draught, under-keel clearance and the general state of the ship, to avoid dangerous listing or trim during cargo handling or ballasting,

- the weather and sea state,

- the observance of all regulations concerning safety and fire protection,

- the water level in bilges and tanks,

- all persons on board and their location, especially those in remote or enclosed spaces, and

- the exhibition and sounding, where appropriate, of lights and signals;

(3) In bad weather, or on receiving a storm warning, take the necessary measures to protect the ship, persons on board and cargo;

(4)  Take every precaution to prevent pollution of the environment by the ship;

(5) In an emergency threatening the safety of the ship, raise the alarm, inform the master, take all possible measures to prevent any damage to the ship, its cargo and persons on board, and, if necessary, request assistance from the       shore authorities or neighboring ships;

| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 5 of 7 |
| --- | --- | --- |

- Be aware of the ship's stability condition so that, in the event of fire, the shore fire-fighting authority may be

advised of the approximate quantity of water that can be pumped on board without endangering the ship;

(7) Offer assistance to ships or persons in distress;

(8) Take necessary precautions to prevent accidents or damage when propellers are to be turned; and

(9) Enter in the appropriate log-book all important events affecting the ship.

5.  Taking over the engineering watch

5.1 Prior to taking over the engineering watch, the relieving officer shall be informed by the officer in charge of the engineering watch as to:

(1) The standing orders of the day, any special orders relating to the ship operations, maintenance functions, repairs to the ship's machinery or control equipment;

(2) The nature of all work being performed on machinery and systems on board ship, personnel involved and potential hazards;

(3) The level and condition, where applicable, of water or residue in bilges, ballast tanks, slop tanks, sewage tanks, reserve tanks and special requirements for the use or disposal of the contents thereof;

(4) Any special requirements relating to sanitary system disposals;

(5) The condition and state of readiness of portable fire-extinguishing equipment and fixed fire-extinguishing installations and fire-detection systems;

(6) Authorized repair personnel on board engaged in engineering activities, their work locations and repair functions and other authorized persons on board and the required crew;

(7) Any port regulations pertaining to ship effluents, fire-fighting requirements and ship readiness, particularly during potential bad weather conditions;

(8) The lines of communication available between the ship and shore personnel, including port authorities, in the event of an emergency arising or assistance being required;

(9) Any other circumstance of importance to the safety of the ship, its crew, cargo or the protection of the environment from pollution; and

(10) The procedures for notifying the appropriate authority of environmental pollution resulting from engineering activities.

5.2 Relieving officers, before assuming charge of the engineering watch, shall satisfy themselves that they are fully informed by the officer being relieved, as outlined above, and:

(1) Be familiar with existing and potential sources of power, heat and lighting and their distribution;

(2) Know the availability and condition of ship's fuel, lubricants and all water supplies; and

(3) Be ready to prepare the ship and its machinery, as far as is possible, for stand-by or emergency conditions as required.

6.  Performing the engineering watch

6.1 Officers in charge of the engineering watch shall pay particular attention to:

(1) The observance of all orders, special operating procedures and regulations concerning hazardous conditions and their prevention in all areas in their charge;

(2) The instrumentation and control systems, monitoring of all power supplies, components and systems in operation;

(3) The techniques, methods and procedures necessary to prevent violation of the pollution regulations of the local authorities; and

(4) The state of the bilges.

6.2 Officers in charge of the engineering watch shall:

(1) In emergencies, raise the alarm when in their opinion the situation so demands, and take all possible measures to prevent damage to the ship, persons on board and cargo;

(2) Be aware of the deck officer's needs relating to the equipment required in the loading or unloading of the cargo and the additional requirements of the ballast and other ship stability control systems;

(3) Make frequent rounds of inspection to determine possible equipment malfunction or failure, and take immediate remedial action to ensure the safety of the ship, of cargo operations, of the port and the environment;

(4) Ensure that the necessary precautions are taken, within their area of responsibility, to prevent accidents or damage to the various electrical, electronic, hydraulic, pneumatic and mechanical systems of the ship;

(5) Ensure that all important events affecting the operation, adjustment or repair of the ship's machinery are satisfactorily recorded.

| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 6 of 7 |
| --- | --- | --- |

7. Search for Stowaway

Complete a thorough stowaway search before departure / arrival per master’s decision and always maintain a gangway watch. A thorough search shall be carried out by ship hands prior to arrival and departure from the port, especially when departing from African ports;

7.1. Steps to prevent stowaways;

- While in port the accommodation doors should be locked or guarded

- A crew member must be duty always on the gangway, tallying all persons boarding and disembarking

- In African countries this gangway duty should, if possible, be shared with African watchman / guards,  who may be able to recognize the nationality of persons trying to board by their appearance

- Stevedore companies should be asked before operations begin how many stevedores will be working and stevedores should be required to access the ship only by the gangway

- Watchman should be vigilant for boarders climbing the fore and aft mooring ropes, and over the rails from the quayside at low water or by small boat, especially during the night

- Specific instructions should be given to watchman to allow on board only people required for the ship's business, such as stevedores and officials. Hawkers should not be permitted

- Before departure, the vessel should be searched thoroughly with particular attention to dark and unlikely places, including areas apparently locked

- All containers provided for loading without seals should be opened, inspected so far as possible and then sealed prior to loading

7.2. Steps to be taken after a stowaway has been detected;

Many stowaways give themselves up once the vessel is at sea, often by making a loud noise. For a vessel discovering stowaways, the priority is for them to be disembarked at the next port of call. The master should therefore immediately inform the owners and the Club or the Club corresponded as soon as possible

A master should, if possible, immediately:

- Search the area where the stowaway was found for concealed papers, etc.

- Search the stowaway's clothing for identification papers, weapons or drugs

- Advise owner or manager immediately

- Interview the stowaway and immediately advise the Member and the Agents at the next port of  call of the following:

- port of embarkation

- details of documented information held

- name

- date and place of birth

- address

- nationality

- The stowaway should be kept secure at all times, particularly when the ship is in port

- While the stowaway is on board, the master should not provide work for him and the stowaway should not  be signed on to the ship's Articles

- The Member should then immediately  advise the Club of the above together with:

- Full itinerary

- Details of agents at future ports of call

- Details of ship's radio/fax/telex

- The Club will agree a course of action with the Member and instruct local correspondents where necessary.

Masters should always bear in mind that stowaways frequently give false details in order to delay their removal from the ship. If the master believes that the stowaway is not telling the truth he should so report

- Do not add stowaways to the crew list

| YA-SA GEMI ISLETMECILIGI VE TICARET A.S. | SHIPBOARD OCCUPATIONAL HEALTH AND SAFETY MANAGEMENT PROCEDURES 1 SHIPBOARD OPERATION PROCEDURES | SHP/OHSMP1/YS-531 Section No: 3.1.2 Issue No: Original Rev. No: 01 Date : 17/10/2022 Page : 7 of 7 |
| --- | --- | --- |

- Documented Information

1) YS-O-30 Procedure For Plan Loading Check List

2) YS-O-31 Prevention For Cargo related Casualities Check List

3) YS-O-32 Duties of the Chief Officer in the Loading Port Check List

4) YS-O-33 Duties of the Officer of the Watch in the Loading Port Check List

5) YS-O-35 Duties of the Chief Officer in the Discharging Port Check List

6) YS-O-36 Duties of the Officer of the Watch in the Discharging Port Check List

7) YS-531-03-08-C Anchor Watch Checklist

8) YS-531-03-10 Navigating/Drifting In Heavy Weather or In Tropical Storm Areas Checklist

9) YS-531-03-06 Changing Over The Watch Check List

10) YS-531-03-14 Stowaway&Suspicious Object  Search Check List